High-performance door ASSA ABLOY RR5000

Description:

High-performance door, opening in vertical direction, for interior and exterior use.

Side frames : galvanized steel profiles + insulation with ASSA ABLOY extrusion. Continuous side guide strip

Door curtain: foam lamellas, fully separated outer sheet metal skin, thickness 50 mm, covered by metal sheet color silver (RAL 9006). doubled sealing system,

Optionally:  window lamellas, double glazed, up to a transparency of 65 %.

Lifting methode: high tech lifting belt

Repair process: individual lamellas, repair time 20min per lamella

Contactless Winding: simple V-Drive Disc Drive technology, spiral disc, no wear

Wind resistance (EN 12424): class 4 (DW ≤ 3500mm), class 3 (3500mm < DW ≤ 5000mm).

Thermal transmittance door blade: according EN 12428: 0,79 W/m²K.

Thermal transmittance complete door: according EN 12428: 1,28 W/m²K.

Noise emission in dB: (ISO 140-3): 25 dB.

Temperature range: -20°C to +40°C.

Drive unit: gear motor 1,1 KW. Chain drive to reduce mounting space to the side is delivery.

Protection: IP 55.

Control system:

MCC frequency control unit, with vector control technology and boost function at start-up. Graphic display and foil keypad for easy guided operation. Easy access to all important information for operation and service

Control voltage 24 V DC

Supply voltage:

3L/(N)PE 380/400/415/440/460/480V; 50/60 Hz

With transformator3L/(N)PE/220/230/500V; 50Hz

Safety devices:

- Light curtain,

- drop down protection by integrated balancing system

Manual activation: by releasing the brake at ground level. Partly opening by means of the pre-stressed tension springs.

Opening / closing speed: up to max. 2.2/0.7 m/s

Size min.: (DW x DH): 1250 x 2500 mm

Size max.: (DW x DH): 5000 x 4700 mm

Type: ASSA ABLOY RR5000

Options

Covers:

- Top roll cover made of anodized aluminum

- Top roll cover powder coated

- Motor cover made of anodized aluminum

- Motor cover powder coated

Window slats:

- Lamellas with transparency window slats Polycarbonat (PC)

Surface/material:

- Standard lamellas: painting in RAL colours (one or both sides)

- Side frames: powder coated in RAL colours

- Top roll cover and motor cover: powder coated in RAL colours

Chain drive:

- Chain drive to reduce mounting space to the side

Hand chain drive:

- A hand chain drive can be installed to open and close the door completely

Detector for damaged door slats:

- A detector recognizing damaged door slats can be installed. The control stops the door blade in case of critical damages.

Safety:

- Contact edge

- Pre-running photo cell is integrated in the side frame instead of a light curtain.

Miscellaneous:

- Actuators