SECTION 08 42 29.43 – IMPACT RATED SLIDING AUTOMATIC ENTRANCES

- GENERAL

  - SUMMARY

- NOTE:  Edit this specification as required for the project and for the impact rating

- NOTE: The SL500 Resilience R128 is available as the following types of entrances:

- Impact rated “Level D” Large and Small Missile Impact (50 feet per second)

- Non-impact entrances.  Non-impact entrances require shutter system for use in windborne debris regions.

- Both State of Florida and Miami-Dade approved configurations are included

- NOTE:  The design pressure of the SL500 Resilience R128 is dependent on the entrance configuration, glass type, and frame size (height x width).  The available design pressures are as follows:

- Bi-Parting, Full Breakout Entrance, Level D Large and Small Missile Impact:  60 psf to 99.5 psf

- Single Slide, Full Breakout Entrance, Level D Large and Small Missile Impact:  60 psf to 114.8 psf

- Bi-Parting, Full Breakout Entrance, Non-Impact:		55 psf to 112 psf

- Single Slide, Full Breakout Entrance, Non-Impact:	48.9 psf to 116.3 psf

- NOTE:  The design pressure of the SL500 Resilience R128 based on the entrance configuration and frame size (height x width) is available at the following link:

- https://www.assaabloyentrance.us/en/products/automatic-doors/automatic-sliding-doors/resilience-automatic-sliding-doors/

- NOTE:  Size of the SL500 Resilience R128 is as follows:

- Bi-Parting Entrance:		Minimum width	120”

- Maximum width	192”

- Single Slide Entrance	Minimum width	80”

- Maximum width	108”

- Frame Height			Maximum height	128”

    - This section includes the following types of automatic entrance doors:

      - Exterior, large and small missile impact, sliding automatic entrances.

        - Approved for HVHZ areas, non-HVHZ areas where windborne debris protection requirements exist, and areas where structural windload requirements exist; all without use of an impact protective system.

      - Exterior, non-impact, sliding automatic entrances.

    - Related Sections:

      - Division 7 Sections for caulking to the extent not specified in this section.

      - [Division 8 Section “Aluminum-Framed Entrances and Storefronts” for entrances furnished separately in Division 8 Section.]

      - [Division 8 Section “Door Hardware” for hardware to the extent not specified in this Section.]

      - Division 8 Section “Glazing” for materials and installation requirements of glazing for automatic entrance doors.

      - Division 26 and 28 Sections for electrical connections including conduit and wiring for automatic entrance door operators and access control devices.

  - REFERENCES

    - References:  Refer to the version year adopted by the Authority Having Jurisdiction.

      - ANSI A117.1 - Accessible and Usable Buildings and Facilities.

      - ICC/IBC - International Building Code.

      - NFPA 70 - National Electrical Code.

      - NFPA 101 - Life Safety Code.

    - American National Standards Institute (ANSI) / Builders Hardware Manufacturers Association (BHMA).

      - ANSI/BHMA A156.10 American National Standard for Power Operated Pedestrian Doors.

      - ANSI Z97.1 Standards for Safety Glazing Material Used in Buildings.

    - Underwriters Laboratories (UL).

      - UL 325 Standard for Safety for Door, Drapery, Gate, Louver and Window Operators and Systems.

    - American Association of Automatic Door Manufacturers (AAADM).

    - American Society for Testing and Materials (ASTM).

      - ASTM B221 Standard Specification for Aluminum and Aluminum Alloy Extruded Bars, Rods, Wire, Profiles and Tubes.

      - ASTM B209 Standard Specification for Aluminum and Aluminum Alloy Sheet and Plate.

      - ASTM E283 – Standard Test Method for Determining the Rate of Air Leakage Through Exterior Windows, Curtain Walls, and Doors Under Specified Pressure Differences Across the Specimen.

      - ASTM E330 – Standard Test Method for Structural Performance of Exterior Windows, Doors, Skylights and Curtain Walls, Doors by Uniform Static Air Pressure Difference. Testing conducted for both positive and negative pressure.

      - ASTM F842 – Standard Test Methods for Measuring the Forced Entry Resistance of Sliding Door Assemblies, Excluding Glazing Impact.

      - ASTM E1886, ASTM E1996 – Standard Test Method for Performance of Exterior Windows, Curtain Walls, Doors, and Impact Protective Systems Impacted by Missile(s) and Exposed to Cyclic Pressure Differentials.

    - American Architectural Manufacturers Association (AAMA).

      - AAMA 611 Voluntary Specification for Anodized Architectural Aluminum.

    - National Association of Architectural Metal Manufacturers (NAAMM).

      - Metal Finishes Manual for Architectural Metal Products.

    - International Code Council (ICC).

      - [IBC:  International Building Code Building Code.]

      - [CBC: California Building Code.]

    - Florida Building Code, 2001, 2004, 2007, 2010, 2014, 2017.

    - Florida Administrative Code (FAC).

      - 9B-72 – Product Approval.

- NOTE:  The following is available with the SL500 Resilience R128 full breakout configuration – Delete if not required

    - [Miami-Dade County Building Code Compliance Office.]

      - Product Control Division, Notice of Approval.

  - DEFINITIONS

    - Activation Device: Device that, when actuated, sends an electrical signal to the door operator to activate the operation of the door.

      - Knowing act:	Consciously initiating the opening of a power operated door using acceptable methods including wall mounted switches such as push plates and controlled access devices such as keypads, card readers and key switches.

    - Safety Device: A device that detects the presence of an object or person within a zone where contact could occur and provides a signal to stop the movement of the door.

    - HVHZ: High-Velocity Hurricane Zone.

  - PERFORMANCE REQUIREMENTS

    - Compliance with the following:

      - ANSI/BHMA A156.10 American National Standard for Power Operated Pedestrian Doors.

      - UL 325 listed.

    - Automatic door equipment accommodates medium to heavy pedestrian traffic.

    - Entrapment Force Requirements:

      - Power Operated Sliding Doors: Not more than 30 lbf (133 N) required to prevent stopped door from closing.

      - Sliding doors provided with a breakaway device shall require no more than 50 lbf (222N) applied 1 inch (25 mm) from the leading edge of the lock stile for the breakout panel to open.

  - SUBMITTALS

    - Product Data:  Manufacturer's product data sheets including installation details, material descriptions, dimensions of individual components and profiles, fabrication, operational descriptions and finishes.

    - Shop Drawings:  Submit manufacturer’s shop drawings, including elevations, sections and details, indicating dimensions, materials, and fabrication of doors, frames, sidelites, operator, motion /presence sensor control device, anchors, hardware, finish, options and accessories.

    - Samples: Submit manufacturer’s samples of aluminum finish.

    - Informational Submittals:  Manufacturer's product information and applicable sustainability program credits that are available to contribute towards a LEED rated project certification.

      - Credit MR 4: Manufacturer's or fabricator's certificate indicating percentage of post-consumer recycled content by weight and pre-consumer recycled content by weight for each Product specified under this Section.

    - Test Reports:  Submit certified test reports indicating doors comply with specified performance requirements.

    - Manufacturers Field Reports:  Submit manufacturer’s field reports from AAADM certified technician of inspection and approval of doors for compliance with ANSI/BHMA A156.10 after completion of installation.

    - Operating and Maintenance Manuals:  Provide manufacturers operating and maintenance manuals for each item comprising the complete door opening installation in quantity as required in Division 01, Closeout Submittals.  The manual to include the name, address, and contact information of the manufacturers providing the hardware and their nearest service representatives.  The final copies delivered after completion of the installation test to include spare parts list.

    - Warranties and Maintenance:  Special warranties and maintenance agreements specified in this Section.

  - QUALITY ASSURANCE

    - Manufacturers Qualifications:  Engage qualified manufacturers with a minimum 10 years of documented experience in manufacturing of doors and equipment of similar to that indicated for this Project and that have a proven record of successful in-service performance.  Manufacturer to have a company certificate issued by AAADM.

    - Installer Qualifications:  Installers, trained by the primary product manufacturers, with a minimum 3 years documented experience installing and maintenance of units similar in material, design, and extent to that indicated for this Project and whose work has resulted in construction with a record of successful in-service performance.

    - Certified Inspector Qualifications: Certified by AAADM.

    - Source Limitations for Automatic Entrances:  Obtain each type of door, frame, operator and sensor components specified in this Section from a single source, same manufacturer unless otherwise indicated.

    - Power-Operated Pedestrian Door Standard: ANSI/BHMA A156.10 (current version).

    - Emergency Exit door requirements: Comply with requirements of authorities having jurisdiction for automatic entrance doors serving as a required means of egress.

  - PROJECT CONDITIONS

    - Field Measurements:  Verify actual dimensions of openings to receive automatic entrances by field measurements before fabrication and indicate on shop drawings.

  - COORDINATION

    - Coordinate sizes and locations of recesses in concrete floors for recessed tracks and thresholds if applicable. Concrete work is specified in Division 03.

    - Electrical System Roughing-in: Coordinate layout and installation of automatic entrances with connections to power supplies and access control system as applicable.

  - WARRANTY

    - General Warranty:  Reference Division 01, General Requirements.  Special warranties specified in this Article shall not deprive Owner of other rights Owner may have under other provisions of the Contract Documents and shall be in addition to, and run concurrent with, other warranties made by Contractor under requirements of the Contract Documents.

    - Automatic Entrance Doors shall be free of defects in material and workmanship for a period of One (1) year from the date of substantial completion.

    - During the warranty period a factory-trained technician shall perform service and affect repairs. An inspection shall be performed after each adjustment or repair.

    - During the warranty period all warranty work, including but not limited to emergency service, shall be performed during normal business hours.

    - Manufacturer shall have in place a dispatch procedure that shall be available 24 hours a Day, 7 Days a week for emergency call back service.

- PRODUCTS

  - MANUFACTURER

    - Manufacturer:  ASSA ABLOY Entrance Systems, 1900 Airport Road, Monroe, NC 28110. Toll Free (877) SPEC-123.  Fax (704 ) 290- 5555 Website: www.assaabloyentrance.us  contact:  specdesk.na.entrance@assaabloy.com

- NOTE:  Revise the following substitution clause as required by project requirements.  Select either Item “B” or “C”

    - [Substitutions:  Requests for substitution and product approval in compliance with the specifications must be submitted in writing and in accordance with the procedures outlined in Division 1, Section “Substitution Procedures”.  Approval of requests is at the discretion of the architect, owner, and their designated consultants.]

    - [Substitutions:  Not Permitted.]

  - SLIDING AUTOMATIC ENTRANCES

    - Sliding automatic entrance system including the following:

      - Sliding panels, sidelites and aluminum frame.

      - Overhead concealed, electro-mechanical operator.

      - Operator housing, guide system and carrier assemblies.

      - Controls and accessories as required for a complete installation.

      - Entrance to have a State of Florida FL number.

- NOTE:  Retain the following for impact rated entrances

      - [Approved for HVHZ areas, non-HVHZ areas where windborne debris protection requirements exist, and areas where structural windload requirements exist; all without use of an impact protective system.]

- NOTE:  The following is available only with the SL500 Resilience R128 full breakout configuration – Delete if not required

      - [Entrance to have a Miami-Dade NOA number (notice of approval).]

    - Performance Requirements:

      - Air Infiltration per TAS 202, ASTM E283 – Standard Test Method for Determining the Rate of Air Leakage Through Exterior Windows, Curtain Walls, and Doors Under Specified Pressure Differences Across the Specimen.

        - Maximum air leakage of 1.0 cfm/ft2 at 1.57 psf (25 mph).

      - Structural Performance (wind load) per TAS 202, ASTM E330 – Standard Test Method for Structural Performance of Exterior Windows, Doors, Skylights and Curtain Walls, Doors by Uniform Static Air Pressure Difference. Testing conducted for both positive and negative pressure.

- NOTE:  The design pressures listed below are the minimum design pressures based on the maximum frame size as listed in the State of Florida Approval – FL30356.  For other design pressures, refer to the State of Florida Approval

- NOTE:  The design pressure of the SL500 Resilience R128 based on the entrance configuration and frame size (height x width) is available at the following link:

- https://www.assaabloyentrance.us/en/products/automatic-doors/automatic-sliding-doors/resilience-automatic-sliding-doors/

        - [Level D Large and Small Missile Impact, single slide, full breakout entrance:  Minimum design pressure not less than 60 PSF.  (Design pressure is based on a 128-inch frame height x 108-inch frame width.)]

          - No more than 0.4” of permanent set will be acceptable.

        - [Level D Large and Small Missile Impact, bi-parting, full breakout entrance:  Minimum design pressure not less than 55 PSF.  (Design pressure is based on a 128-inch frame height x 192-inch frame width.)]

          - No more than 0.4” of permanent set will be acceptable.

        - [Non-Impact, single slide, full breakout entrance:  Minimum design pressure not less than 48.9 PSF.  (Design pressure is based on a 128-inch frame height x 108-inch frame width.)]

          - No more than 0.4” of permanent set will be acceptable.

        - [Non-Impact, bi-parting, full breakout entrance:  Minimum design pressure not less than 60 PSF.  (Design pressure is based on a 128-inch frame height x 192-inch frame width.)]

          - No more than 0.4” of permanent set will be acceptable.

      - Forced Entry Resistance per FBC TAS 202, ASTM F842 – Standard Test Methods for Measuring the Forced Entry Resistance of Sliding Door Assemblies, Excluding Glazing Impact.

        - Must pass Grade 25 with No Entry.

- NOTE:  Retain the following for impact rated entrances that meet Level D Large and Small Missile Impact

      - [Large and Small Missile Impact Test per TAS 201, ASTM E1886, ASTM E1996 – Standard Test Method for Performance of Exterior Windows, Curtain Walls, Doors, and Impact Protective Systems Impacted by Missile(s) and Exposed to Cyclic Pressure Differentials.]

        - Level D Large and Small Missile Impact (50 feet per second).

      - Cyclical Test per TAS 203, ASTM E 1996.

      - State of Florida Approval Numbers:

        - Full Breakout, Level D Large and Small Missile Impact – FL30356.R5.

        - Full Breakout, Non-Impact – FL30356.R5.

- NOTE:  Delete the following if not required

      - [Miami-Dade Product Control Approved.]

        - Full Breakout, Level D Large and Small Missile Impact – NOA 21-0324.12.

        - Full Breakout, Non-Impact – NOA 21-0324.13.

- NOTE:    Select entrance configuration(s) as required for the project

- NOTE:  Fixed sidelite configurations are not available with the SL500 Resilience R128

    - ASSA ABLOY SL500 Resilience R128 Sliding Automatic Entrance (Basis of Design) with Stile and Rail Panels:

      - [Impact resistant, single slide, full breakout, door system.]

        - Configuration:  Single slide, two equal panel unit with one operable leaf and one sidelite.

        - Traffic Pattern:  [Two-way.]  [One-way.]

        - Emergency Breakaway Capability:  Sliding leaf and sidelite.

        - Mounting:  Overhead header installed between jambs.

        - Impact Rating:  Level D Large and Small Missile Impact (50 feet per second).

      - [Non-impact, single slide, full breakout, door system.]

        - Configuration:  Single slide, two equal panel unit with one operable leaf and one sidelite.

        - Traffic Pattern:  [Two-way.]  [One-way.]

        - Emergency Breakaway Capability:  Sliding leaf and sidelite.

        - Mounting:  Overhead header installed between jambs.

        - Impact Rating:  Non-Impact.

      - [Impact resistant, bi-parting, full breakout, door system.]

        - Configuration:  Bi-parting, four equal panel unit with two operable leaves and two sidelites.

        - Traffic Pattern:  [Two-way.]  [One-way.]

        - Emergency Breakaway Capability:  Sliding leaves and sidelites.

        - Mounting:  Overhead header installed between jambs.

        - Impact Rating:  Level D Large and Small Missile Impact (50 feet per second).

      - [Non-impact, bi-parting, full breakout, door system.]

        - Configuration:  Bi-parting, four equal panel unit with two operable leaves and two sidelites.

        - Traffic Pattern:  [Two-way.]  [One-way.]

        - Emergency Breakaway Capability:  Sliding leaves and sidelites.

        - Mounting:  Overhead header installed between jambs.

        - Impact Rating:  Non-Impact.

  - ENTRANCE COMPONENTS

    - Stile and Rail Sliding Panels and Sidelites:

      - Material:  Extruded Aluminum, Alloy 6063-T5.

      - Door panels shall have a minimum .125 inch (3.2 mm) structural wall thickness including adjoining horizontal members and perimeter frames where applicable.

      - Door construction shall be by means of concealed steel reinforcement and self-aligning aluminum clips with 3/8 inch all-thread through bolt from each stile.

      - Glass stops shall be 0.090” (2.79mm) wall thickness.

      - Full breakout sliding entrances shall include two interlocks per moving panel securing the leading stile of the sidelite and the butt stile of the sliding door panel together.

        - Interlocks to be automatic engaging and releasing during open-close cycles.

- NOTE:  Narrow stiles are only available for non-impact entrances; medium stiles are available for both impact and non-impact entrances

      - Vertical Stiles shall be [narrow stile 2-1/8 inch (54 mm).]  [medium stile 4 inch (102 mm).]

      - Bottom Rails shall be 10 inch (254 mm).

- NOTE:  The lower intermediate muntin is required in either a 1-3/4 inch or 4 inch width

      - Lower Intermediate Muntin width shall be [1-3/4 inch (45 mm).]  [4 inch (102 mm).]

- NOTE:  The upper intermediate muntin is optional, but must match the width of the lower intermediate muntin

      - [Upper Intermediate Muntin shall match the width of the lower intermediate muntin.]

      - Weather-seals:   Slide-in replaceable, heavy pile weather-seals.  Complementing weather-seals shall be provided on joining trailing and lead edge vertical stiles.  Heavy pile weather-seals shall be installed between the carrier and active leaf(s) top rail. Self-adhesive bulb seal shall be provided between the header and top rail of sidelites.  Bottom rails shall be provided with dual concealed adjustable nylon sweep.

- NOTE:  Select glazing types as required for the project.  Add “Locations” to each of the following if multiple types and/or thickness glazing is required.   Ex: Exterior vs Interior

      - Glass:  Glazing shall comply with ANSI Z97.1, thickness as indicated.

        - [Glazing, Level D Large and Small Missile Impact Rated Entrances:  9/16” overall thickness, laminated impact glass consisting of an approved interlayer laminated between two pieces of 1/4 inch (6mm) heat strengthened glass panes.]

          - Approved Interlayers:

            - .090” SentryGlas Plus® by DuPont.

- NOTE:  Select above for impact rated entrances or select  below  for non-impact entrances

        - [Glazing Non-Impact Entrances:  1/4” (6 mm) tempered glass.]

        - Glazing Installation:  Refer to the sliding entrance manufacturer instructions to meet the specified performance of the sliding entrance.

          - [All Glazing furnished “by others”.]

          - Door panels must be dry glazed using a captive seal and press-in wedge seal.

            - Glazing installation with silicone sealant is not acceptable.

          - No exposed fasteners in the glazing stops will be acceptable.

    - Door Carriers:  Manufacturer’s special carrier assembly for impact rated sliding entrances that allows vertical adjustment.

      - Sliding Panel Door Carriers:

        - Roller Wheels:  Four (4) plastic wheels, 2 inch diameter.  Single journal with sealed oil impregnated bearings.

        - Two (2) self-aligning anti-risers per leaf.

    - Framing Members:  Provide automatic entrances as complete assemblies.  Manufacturer’s standard extruded aluminum framing reinforced as required to support loads.

      - Sliding Entrance - Vertical Jambs:  1-3/4 inches (44.5 mm) by 4-1/2 inches (114.3 mm).

    - Header:  Manufacturer’s standard extruded aluminum header with a replaceable aluminum track extending full width of entrance unit.  Header to conceal door operators, carrier assemblies, and roller track; complete with hinged access panel for service of door operator, and controls.

      - Sliding Entrance - Header Span:   Maximum 16’-0” (4.9 m) when installed per manufacturer’s recommendations.

        - Capacity:  Capable of supporting active breakout leafs up to maximum of 400 lbs (181kg) per leaf when header is supported per manufacturer’s recommendations.

        - Size:  4-1/2 inches (114.3 mm) wide by 7 inches (177.8 mm) high.

          - Header height including the sensor plate cap which spans the clear door opening width is 9 inches (228.6mm) high.

      - Header Access:  Continuous hinge at top of header allows for complete access to operator and internal electronic and mechanical assemblies.

      - Design:  Closed header when doors in closed position.

  - HARDWARE

    - Hardware:  Provide manufacturer’s standard hardware as required for operation indicated.

      - Breakaway arms and bottom pivot assemblies shall be supplied by the manufacturer and shall be adjustable to comply with applicable codes.

- NOTE:  The optional dampers control the doors in the breakout function

      - [Wind resistant hydraulic 90 degree damper to control movement of breakout panels.]

- NOTE:  Consult SpecDesk for locking hardware options and hardware for access control

      - Locking hardware shall be provided as indicated.

        - [Electrified slide lock shall automatically lock the sliding function of all sliding door panels within the entrance when the door panels are in the closed position.]

          - [Fail secure operation:  Slide lock shall lock the sliding function of the door panels upon loss of power.]

          - [Fail safe operation:  Slide lock shall unlock the sliding function of the door panels upon loss of power.]

          - Exterior jamb mounted key switch to unlock sliding door operation.

- NOTE:  The following manual locking system is required for single slide entrances

        - [Three point locking system on each active leaf with hookbolt latch and throw rod/solid steel bolts into carrier arm and into threshold.]

          - Locking System:  Locking shall not require more than one operation for release of each active panel.

          - Interior Side: [Thumbturn.]  [Keyed cylinder.]  [Adams-Rite 4550 lever.]

            - Lock indicators shall be provided.

          - Exterior Side: [Keyed cylinder.]  [No cylinder.]

          - Armored steel reinforcement as required.

- NOTE:  The following manual locking system is required for bi-parting entrances

        - [Two point locking system on each active leaf with throw rod/solid steel bolts into carrier arm and into threshold.]

          - Locking System:  Locking shall not require more than one operation for release of each active panel.

          - Interior Side: [Thumbturn.]  [Keyed cylinder.]  [Adams-Rite 4550 lever.]

            - Lock indicators shall be provided.

          - Exterior Side: [Keyed cylinder.]  [No cylinder.]

          - Armored steel reinforcement as required.

      - Keyed cylinders shall be provided as indicated.

        - [Manufacturer’s standard keyed cylinder.]

        - [Keyed cylinder specified in Division 8 Section “Door Hardware”.]

        - [Keyed cylinder by others.]

    - Guide Track/Threshold:  Manufacturer’s threshold as indicated.

      - Full Breakout Entrance Threshold:  1/2 inch (12.7 mm) high continuous aluminum threshold with integral track shall span the width of the sliding door header and fit between the vertical framing members.  Threshold design shall allow for optional extruded ramps to securely interlock to flat section to meet ADA requirements.

        - [Surface mounted threshold with interlocking ADA accessible ramps.]  [Recessed mounted threshold.]

  - DOOR OPERATORS AND CONTROLS

    - Door Operator and Controller:

      - Electro-mechanical controlled unit utilizing a high-efficiency, energy efficient, DC motor requiring a maximum of 3 amp current draw, allowing 5 operators on one 20 amp circuit. The supplied system shall have the capability to operate at full performance well beyond a brown out and high line voltage conditions (85V – 265V) sensing changes and adjusting automatically.  The operator shall allow an adjustable hold open time delay of 0 to 60 seconds and have internal software to incorporate a self-diagnostic system.

        - Operating Temperature Range: -31 F to 130 F (-35 C to 54.44 C).

    - Microprocessor Control Box:

      - Modular control unit to allow for changing technology.  Factory-adjusted configuration with opening and closing speeds set to comply with ANSI/BHMA A156.10 requirements and electronic dampening to reduce wear on drive train. Should the drive train operations deviate from design criteria ranges, Watchdog Control Circuit Monitoring will assume command of the system and shut down the automatic function allowing a secondary supervisory circuit to perform as a backup.  Control unit shall allow the following functions:

        - Diagnostics with the ability to produce application data.

      - Mode Selector Control:

- NOTE:  The standard mode selector control is the rotary knob switch

        - Multi-position [rotary knob] [keyed cylinder] mode selector control shall allow selection of the indicated functions to be engaged when switch is turned to the appropriate setting.

- NOTE:  The standard mounting location is jamb mounted

        - Mode Selector Control Mounting:  Control shall be mounted as indicated:

          - [Jamb mounted.]

          - [Remote mounted.]

        - Mode selector control to allow the following functions:

          - “Off”

          - “Exit Only” one way traffic with automatic operation from the interior.

          - “Two Way Traffic” allowing automatic operation from exterior and interior.

          - “Partial Opening” energy saving door position allows door to only open partially upon activation from exterior and interior.

          - “Hold Open” doors activated and held in the full open position.

  - ACTIVATION AND SAFETY CONTROL DEVICES

    - General:  Provide the types of activation and safety devices specified in accordance with ANSI/BHMA standards, for the condition of exposure and for long-term, maintenance-free operation under normal traffic load for type of occupancy indicated.  Coordinate activation and safety devices with door operation and door operator mechanisms.

    - Combination Activation Motion Sensor/Safety Presence Sensor:

      - Shall be a sliding door sensor utilizing K-band microwave technology to detect motion and focused active infrared technology to detect presence, combined in a single housing surface mounted on each side of the header.

        - Presence sensor shall remain active at all times.

        - The sensor shall communicate with the automatic door operator through a self-monitoring connection that allows the door to go into a fail-safe mode preventing the door from closing in the event of a sensor failure.

      - Motion/presence detecting sensors to be field installed and adjusted.

- NOTE:  Consult Besam for “Knowing Act Activation” device options.  Ex: push plates

  - ELECTRICAL

    - High-Efficiency DC Motor: Maximum of 3 amp current draw, allowing 5 operators to run on one 20 Amp circuit.

    - Power:  Self-detecting line voltage capable control. 120 VAC through 240 VAC, 50/60 Hz, 3 amp minimum incoming power with solid earth ground connection for each door system.

    - Wiring:  Separate internal channel raceway free from moving parts.

    - Brown out / high voltage capability: System has capability to operate at full performance well beyond brown out and high voltage line conditions (85 V – 265 V) sensing changes and adjusting automatically.

    - [Convenience Battery: Shall be concealed in header and capable of full operation with blackout conditions, including sensor capabilities for minimum of 100 cycles.]

    - [Digital Cycle Counter:  Battery powered, 7 digit LCD cycle counter with a reset feature to track door usage cycles.]

  - ALUMINUM FINISHES

- NOTE:  Consult SpecDesk for custom finish options

    - Comply with NAAMM's "Metal Finishes Manual for Architectural and Metal Products" for recommendations for applying and designating finishes.

    - [Anodized Finish:]

      - [AAMA 611, Clear, AA- M12C22A41, Class I, 0.018 mm.]

      - [AAMA 611, Dark Bronze, AA-M12C22A44, Class I, 0.018 mm.]

      - [AAMA 611, Custom anodized to match architect’s sample.]

    - [Painted Finish:]

      - [Powder coat painted to match architect’s sample.]

      - [Kynar finish, [2 coat]  [3 coat], to match architect’s sample.]

    - [To match architects sample.]

- EXECUTION

  - EXAMINATION

    - Examine doors and frames, with Installer present, for compliance with requirements for installation tolerances, wall and floor construction, and other conditions affecting performance.

    - Examine roughing-in for electrical source power to verify actual locations of wiring connections.

    - Proceed only after such discrepancies or conflicts have been resolved.

  - INSTALLATION

    - Do not install damaged components.  Fit frame joints to produce hairline joints free of burrs and distortion. Rigidly secure non-movement joints.

    - Entrances:  Install automatic entrances plumb and true in alignment with established lines and grades without warp or rack of framing members and doors.  Anchor securely in place.

      - Install surface mounted hardware using concealed fasteners to greatest extent possible.

      - Set headers, carrier assemblies, tracks, operating brackets and guides level and true to location with anchorage for permanent support.

        - Install entrance with a maximum 1/4" shim space as required by state and local hurricane product approvals.

    - Door Operators:  Connect door operators to electrical power distribution system as specified in Division 26 Sections.

    - Glazing:  Glaze sliding automatic entrance with impact resistant glazing in accordance with the Glass Association of North America (GANA) Glazing Manual, published recommendations of glass product manufacturer, and published instructions of automatic entrance system manufacturer.

    - Sealants:  Comply with requirements specified in division 7 Section “Joint Sealants” to provide a weather tight installation.

      - Set thresholds, bottom guide and track systems and framing members in full bed of sealant.

      - Seal perimeter of framing members with sealant.

    - Signage:  Apply signage on both sides of each door and sidelite as required by ANSI/BHMA A156.10 and manufacturers installation instructions.

  - ADJUSTING

    - Adjust door operators, controls and hardware for smooth and safe operation and for weather tight closure. Adjust doors in compliance with ANSI/BHMA A156.10.

  - FIELD QUALITY CONTROL

    - Before placing doors into operation, AAADM certified technician shall inspect and approve doors for compliance with ANSI/BHMA A156.10. Certified technician shall be approved by the manufacturer.

  - CLEANING AND PROTECTION

    - Clean adjacent surfaces soiled by door installation.

    - Clean glass and metal surfaces promptly after installation. Remove excess sealants, compounds, dirt and other substances. Repair damages to match original finish.

  - DEMONSTRATION

    - Engage a factory-authorized representative to train Owner's maintenance personnel to adjust, operate, and maintain safe operation of the door.

END OF SECTION