High-performance door ASSA ABLOY RR3000 Vision

Description:

High-performance door with transparent PVC sections, opening in vertical direction for interior and exterior use.

Side frames of edged galvanized steel profiles, springs for curtain tension and counterbalance in the side frames.

Door curtain: extensive, transparent PVC sections, framed by anodized aluminum profiles

Wind resistance (EN12424): class 4, class 3 for doors DW > 3500 mm

Temperature range max.: -200 C bis + 600 C

Drive unit: Gear motor

Protection: IP 55

Control system:

MCC, with vector control technology and boost function at start-up. Graphic display and foil keypad for easy guided operation. Easy access to all information for operation and service

control voltage 24 V DC.

Supply voltage:

3L/(N)PE 380/400/415/440/460/480V; 50/60 Hz

With transformator 220/230/500V; 50 Hz

Safety devices:

- patented, contact-free pre-running safety photocell

- door line photocell

- drop down protection by integrated balancing system

Manual activation:

by releasing the brake at ground level. Partly opening by means of the pre-stressed tension springs.

Opening / closing speed: up to max. 1.8/0.7 m/s

Size max.:

(DW x DH) min: 600 x 2525 mm

(DW x DH) min: 5000 x 5000 mm

Make: ASSA ABLOY RR3000 Vision

Options

Covers:

- Top roll cover made of anodized aluminum

- Top roll cover powder coated

- Motor cover made of anodized aluminum

- Motor cover powder coated

Surface/material:

- Side frames RAL powder coated

Miscellaneous:

- extended flexible seal

- Chain drive

Actuators