High-performance door ASSA ABLOY RR300

Description:

Flexible High-performance door, opening in vertical direction for interior use

in wind-protected areas.

Self-supporting side frames made of galvanized steel, all fittings galvanised.

Door curtain made of transparent PVC material with coated fabric reinforcement stripes, ultramarine (similar to RAL 5002) or red (similar to RAL 3002).

Top roll made of aluminium.

Wind protection: through wind tabs on the door curtain.

Wind resistance: class 1 (EN 12424).

Bottom profile: anodized aluminium.

Drive unit: Hollow shaft gear motor, 0,65 kW.

Protection: IP 54.

Control system: MCS-R .

Supply voltage: 3/(N)PE 400V, 50 Hz.

Safety devices:

- Electrical safety contact edge

- Door line photocell

- Drop down protection integrated in gear box

Manual activation: by means of a crank.

Opening / closing speed: up to max. 2.0 / 1.0 m/s.

Size max.:

(DW min. / max.) 1000 mm / 4000 mm

(DH min. / max.)  1000 mm / 4200 mm

Make: ASSA ABLOY RR300

Options

Surface / Material:

- Side frame made of stainless steel

- Control box made of stainless steel V2A - IP 65

- Top roll / shafts made of stainless steel V2A

- Bottom profile made of stainless steel V2A

- Bottom profile “Flex-Edge”

Cover:

- Top roll cover

- Motor cover

Door curtain (alternatively):

- PVC with reinforcement stripes in color:

- Orange (similar RAL 2011)

- Grey (similar RAL 7035)

- Yellow (similar RAL 1021)

- Green (similar RAL 6018)

- Beige (similar RAL 1014)

- White (similar RAL 9016)

- PVC silicone-free reinforcement stripes in color:

- Blue (similar RAL 5010)

- Red (similar RAL 3002)

- Orange (similar RAL 2011)

- Grey (similar RAL 7037)

- Yellow (similar RAL 1021)

- White (similar RAL 9010)

- Black (similar RAL 9005)

- RollTex in color:

- Blue (similar RAL 5002)

- Orange (similar RAL 2009)

- Grey (similar RAL 7001)

- NomaTex in color:

- Blue (similar RAL 5005)

- Red (similar RAL 3000)

- Orange (similar RAL 2009)

- Grey (similar RAL 7038/7045)

- Yellow (similar RAL 1023)

- Top-Curtain 0,8 mm (max. 3000 x 3000 mm) in color:

- Blue (similar RAL 5002)

- Red (similar RAL 3002)

- Yellow (similar RAL 1003)

- Beige (similar RAL 1014)

- Top-Behang 1,3 mm (max. 3000 x 3500 mm) in color:

- Blue (similar RAL 5010)

- Red (similar RAL 3002)

- Grey (similar RAL 7035)

- Window (DWxDH) 500x700mm (only for RollTex® or NomaTex®)

- Window (DWxDH) 700x500mm (only for RollTex® or NomaTex®)

- Vision panel for NomaTex, Top-Curtain 0,8 mm and Top curtain 1,3 mm

Control system (alternatively)

MCC powerful frequency converter control system with Vectoring of the drive unit, connected to the wide range of functions, enables the complex application of the door. The MCC guarantees a soft start, optimal

increase in speed, maximum speeds and a long service life of the door.

- Open/close up to max. 2.0/1.0 m/s, 
- Supply voltage 3L(N)PE/380/400/415/440/480V;50/60Hz, 
   with transformer when 3L/(N)PE/220/230/500V, 50/60Hz

- ACS 50 contactor control system
- Open/close up to max. 1.0/1.0 m/s, 
- Supply voltage 3L/(N)PE/220/230/380/400/415V;50Hz 
- With transformator: 3L/(N)PE/440/480/500V;50 Hz

- MCS-FC frequency converter control system with integrated disply
- Open/Close 1.7/0.8 m/s
- Supply voltage 1/(N)PE 230 50-60 Hz

Bottom profile:

- Model in stainless steel

- Model with bottom profile Flex Edge (flexible bottom profile without aluminum extrusion and coiled cable)

Safety features:

- Crah-Systm at bottom profile with sensor ab B>/= 1300 mm

- Additional safety devices

Miscellaneous:

- Bottom profile with extended flexible seal

- Haspelkette

- Gear heating

- LED traffic light

- Lintel seal

Alternatively:

Version ”Mini” for reduced space requirements over the lintel

- DW min. / max. 500 mm / 3500 mm

- DH min. / max. 500 mm / 3500 mm

- Wind resistance class 0 (EN 12424)

- Due to limited weight of door curtain no drop down protection required

- Open/Close up to max. 1.0/1.1 m/s