SECTION 08 42 29.23 – SLIDING AUTOMATIC ENTRANCES

- GENERAL

  - SUMMARY

- NOTE:  Edit this specification as required for the project

    - This section includes the following types of automatic entrance doors:

      - Interior sliding automatic entrances for Clean Room applications.

    - Related Sections:

      - Division 7 Sections for caulking to the extent not specified in this section.

      - Division 8 Section “Glazing” for materials and installation requirements of glazing for automatic entrance doors.

      - Division 26 and 28 Sections for electrical connections including conduit and wiring for automatic entrance door operators and access control devices.

  - REFERENCES

    - References:  Refer to the version year adopted by the Authority Having Jurisdiction.

      - ANSI A117.1 - Accessible and Usable Buildings and Facilities.

      - ICC/IBC - International Building Code.

      - CUL – Approved for use in Canada.

      - NFPA 70 - National Electrical Code.

      - NFPA 101 - Life Safety Code.

    - American National Standards Institute (ANSI) / Builders Hardware Manufacturers Association (BHMA).

      - ANSI/BHMA A156.10 American National Standard for Power Operated Pedestrian Doors.

      - ANSI Z97.1 Standards for Safety Glazing Material Used in Buildings.

    - Underwriters Laboratories (UL).

      - UL 325 Standard for Safety for Door, Drapery, Gate, Louver and Window Operators and Systems.

    - [Canadian Standards Association (CSA).]

      - CAN/CSA-C22.2 No. 247 – Operators and Systems of Doors, Gates, Draperies, and Louvers.

    - American Association of Automatic Door Manufacturers (AAADM).

    - American Society for Testing and Materials (ASTM).

      - ASTM B221 Standard Specification for Aluminum and Aluminum Alloy Extruded Bars, Rods, Wire, Profiles and Tubes.

      - ASTM B209 Standard Specification for Aluminum and Aluminum Alloy Sheet and Plate.

    - American Architectural Manufacturers Association (AAMA).

      - AAMA 611 Voluntary Specification for Anodized Architectural Aluminum.

    - National Association of Architectural Metal Manufacturers (NAAMM).

      - Metal Finishes Manual for Architectural Metal Products.

    - International Code Council (ICC).

      - [IBC:  International Building Code.]

      - [CBC: California Building Code.]

  - DEFINITIONS

    - Activation Device: Device that, when actuated, sends an electrical signal to the door operator to activate the operation of the door.

      - Knowing act:	Consciously initiating the opening of a power operated door using acceptable methods including wall mounted switches such as push plates and controlled access devices such as keypads, card readers and key switches.

    - Safety Device: A device that detects the presence of an object or person within a zone where contact could occur and provides a signal to stop the movement of the door.

  - PERFORMANCE REQUIREMENTS

    - Compliance with the following:

      - ANSI/BHMA A156.10 American National Standard for Power Operated Pedestrian Doors.

      - UL 325 listed.

    - Automatic door equipment accommodates medium to heavy pedestrian traffic.

    - Automatic door equipment that has been tested and approved for use in an ISO 3 (Class 1) clean room environment.

    - Entrapment Force Requirements:

      - Power Operated Sliding Doors: Not more than 30 lbf (133 N) required to prevent stopped door from closing.

      - Sliding doors provided with a breakaway device shall require no more than 50 lbf (222N) applied 1 inch (25 mm) from the leading edge of the lock stile for the breakout panel to open.

  - SUBMITTALS

    - Product Data:  Manufacturer's product data sheets including installation details, material descriptions, dimensions of individual components and profiles, fabrication, operational descriptions and finishes.

    - Shop Drawings:  Submit manufacturer’s shop drawings, including elevations, sections and details, indicating dimensions, materials, and fabrication of doors, frames, sidelites, operator, motion /presence sensor control device, anchors, hardware, finish, options and accessories.

    - Samples: Submit manufacturer’s samples of aluminum finish.

    - Informational Submittals:  Manufacturer's product information and applicable sustainability program credits that are available to contribute towards a LEED rated project certification.

      - Credit MR 4.1 and 4.2: Manufacturer's or fabricator's certificate indicating percentage of post-consumer recycled content by weight and pre-consumer recycled content by weight for each Product specified under this Section.

    - Manufacturers Field Reports:  Submit manufacturer’s field reports from AAADM certified technician of inspection and approval of doors for compliance with ANSI/BHMA A156.10 after completion of installation.

    - Operating and Maintenance Manuals:  Provide manufacturers operating and maintenance manuals for each item comprising the complete door opening installation in quantity as required in Division 01, Closeout Submittals.  The manual to include the name, address, and contact information of the manufacturers providing the entrance and their nearest service representatives.  The final copies delivered after completion of the installation test to include spare parts list.

    - Warranties and Maintenance:  Special warranties and maintenance agreements specified in this Section.

  - QUALITY ASSURANCE

    - Manufacturers Qualifications:  Engage qualified manufacturers with a minimum 10 years of documented experience in manufacturing of doors and equipment of similar to that indicated for this Project and that have a proven record of successful in-service performance.  Manufacturer to have a company certificate issued by AAADM.

    - Installer Qualifications:  Installers, trained by the primary product manufacturers, with a minimum 3 years documented experience installing and maintenance of units similar in material, design, and extent to that indicated for this Project and whose work has resulted in construction with a record of successful in-service performance.

    - Certified Inspector Qualifications: Certified by AAADM.

    - Source Limitations for Automatic Entrances:  Obtain each type of door, frame, operator and sensor components specified in this Section from a single source, same manufacturer unless otherwise indicated.

    - Power-Operated Pedestrian Door Standard: ANSI/BHMA A156.10 (current version).

    - Emergency Exit door requirements: Comply with requirements of authorities having jurisdiction for automatic entrance doors serving as a required means of egress.

  - PROJECT CONDITIONS

    - Field Measurements:  Verify actual dimensions of openings to receive automatic entrances by field measurements before fabrication and indicate on shop drawings.

  - COORDINATION

    - Coordinate sizes and locations of recesses in concrete floors for recessed tracks and thresholds if applicable. Concrete work is specified in Division 03.

    - Electrical System Roughing-in: Coordinate layout and installation of automatic entrances with connections to power supplies and access control system as applicable.

  - WARRANTY

    - General Warranty:  Reference Division 01, General Requirements.  Special warranties specified in this Article shall not deprive Owner of other rights Owner may have under other provisions of the Contract Documents and shall be in addition to, and run concurrent with, other warranties made by Contractor under requirements of the Contract Documents.

    - Automatic Entrance Doors shall be free of defects in material and workmanship for a period of One (1) year from the date of substantial completion.

    - During the warranty period a factory-trained technician shall perform service and affect repairs. An inspection shall be performed after each adjustment or repair.

    - During the warranty period all warranty work, including but not limited to emergency service, shall be performed during normal business hours.

    - Manufacturer shall have in place a dispatch procedure that shall be available 24 hours a Day, 7 Days a week for emergency call back service.

- PRODUCTS

  - MANUFACTURER

    - Manufacturer:  ASSA ABLOY Entrance Systems, 1900 Airport Road, Monroe, NC 28110. Toll Free (877) SPEC-123.  Fax (704 ) 290- 5555 Website www.assaabloyentrance.us contact: specdesk.na.entrance@assaabloy.com

- NOTE:  Revise the following substitution clause as required by project requirements.  Select either Item “B” or “C”

    - [Substitutions:  Requests for substitution and product approval in compliance with the specifications must be submitted in writing and in accordance with the procedures outlined in Division 1, Section “Substitution Procedures”.  Approval of requests is at the discretion of the architect, owner, and their designated consultants.]

    - [Substitutions:  Not Permitted.]

  - SLIDING AUTOMATIC ENTRANCES

    - Sliding automatic entrance system including the following:

      - Sliding panels, sidelites and aluminum frame.

      - Overhead concealed, electro-mechanical operator.

      - Operator housing, guide system and carrier assemblies.

      - Controls and accessories as required for a complete installation.

- NOTE:  Select entrance configuration(s) as required for the project

    - Besam SL500 Clean Room (Basis of Design) Automatic Sliding Entrance with Stile and Rail Panels:

      - [Single slide, full breakout, door system.]

        - Configuration:  Single slide, two equal panel unit with one operable leaf and one sidelite.

        - Traffic Pattern:  [Two-way.]  [One-way.]

        - Emergency Breakaway Capability:  Sliding leaf and sidelite.

        - Mounting:  Overhead header installed between jambs.

      - [Single slide, fixed sidelite, door system.]

        - Configuration:  Single slide, two equal panel unit with one operable leaf and one fixed sidelite.

        - Traffic Pattern:  [Two-way.]  [One-way.]

        - Emergency Breakaway Capability:  Sliding leaf only.

        - Mounting:  Overhead header installed between jambs.

      - [Single slide, surface mounted, door system.]

        - Configuration:  Single slide unit without sidelite; using a fixed sidelite guide track.

        - Traffic Pattern:  [Two-way.]  [One-way.]

        - Emergency Breakaway Capability:  Sliding leaf.

        - Mounting:  Surface mounted header installed on face of wall.

      - [Bi-parting, full breakout, door system.]

        - Configuration:  Bi-parting, four equal panel unit with two operable leaves and two sidelites.

        - Traffic Pattern:  [Two-way.]  [One-way.]

        - Emergency Breakaway Capability:  Sliding leaves and sidelites.

        - Mounting:  Overhead header installed between jambs.

      - [Bi-parting, fixed sidelite, door system.]

        - Configuration:  Bi-parting, four equal panel unit with two operable leaves and two fixed sidelites.

        - Traffic Pattern:  [Two-way.]  [One-way.]

        - Emergency Breakaway Capability:  Sliding leaves only.

        - Mounting:  Overhead header installed between jambs.

      - [Bi-parting, surface mounted, door system.]

        - Configuration:  Bi-parting, two equal panel unit with two operable leaves and no sidelites; using fixed sidelite guide tracks.

        - Traffic Pattern:  [Two-way.]  [One-way.]

        - Emergency Breakaway Capability:  Sliding leaves.

        - Mounting:  Surface mounted header installed on face of wall.

  - ENTRANCE COMPONENTS

    - Stile and Rail Sliding Panels and Sidelites:

      - Material:  Extruded Aluminum, Alloy 6063-T5.

      - Door panels shall have a minimum .125 inch (3.2 mm) structural wall thickness including adjoining horizontal members and perimeter frames where applicable.

      - Door Construction shall be by means of an integrated corner block with 3/8 inch all-thread through bolt from each stile.

      - Glass stops shall be .062 inch (15.8 mm) wall thickness and shall provide security function as a standard by means of a fixed non-removable exterior section with glazing to be performed from the interior only.

        - 45 degree sloped horizontal glass stops.

      - Full breakout sliding entrances shall include two interlocks per moving panel securing the leading stile of the sidelite and the butt stile of the sliding door panel together.

      - Vertical Stiles shall be [narrow stile 2-1/8 inch (54 mm).]  [medium stile 4 inch (102 mm).]  [wide stile 5 inch (127 mm).]

      - Bottom Rails shall be [4 inch (102 mm).]  [7 inch (178 mm).]  [10 inch (254 mm).]

      - [Intermediate Muntin shall be  [1-3/4 inch (45 mm).]  [4 inch (102 mm).]

      - Gasketing:  Slide-in type, replaceable pile non-shedding Santoprene seals retained by the aluminum extrusions.  The following types of gasketing are required:  complementing gasketing on the joining vertical stiles of the sidelite and sliding door panels, complementing gasketing on the lead edge of the lock stiles of bi-parting doors, gasketing between the carrier and the header, gasketing on the lead edge stile of single slide door panels, gasketing on the pivot stile of breakout sidelite panels, and gasketing on the butt stile of fixed sidelite panels.

      - Clean Room Entrances:  Automatic door equipment that has been tested and approved for use in an ISO 3 (Class 1) clean room environment.

      - Glass:  Glazing shall comply with ANSI Z97.1, thickness as indicated.

        - Glazing Sliding Panels and Sidelite Panels:  1/4” (6 mm) tempered glass.

        - [Glazing Transom Panel:  1/4” (6 mm)] safety glass.]

        - Glazing Installation:  See Division 8 Section “Glazing” for requirements.

          - [All Glazing furnished “by others”.]

    - Door Carriers:  Manufacturer’s standard carrier assembly that allows vertical adjustment.

      - Carriage Assembly:  Carriage bar with two wheel assemblies.  Each assembly shall have tandem roller wheels.

      - Roller Wheels:  Two heavy duty Delrin roller wheels per wheel assembly, for a total of four (4) roller wheels, 1-7/16 inch (36.51 mm) diameter, per active door leaf for operation over a replaceable aluminum track. Single journal with sealed oil impregnated bearings.

      - Two (2) heavy duty self-aligning anti-risers per leaf.

    - Framing Members:  Provide automatic entrances as complete assemblies.  Manufacturer’s standard extruded aluminum framing reinforced as required to support loads.

      - Vertical Jambs:  1-3/4 inches (44.5 mm) by 4-1/2 inches (114.3 mm).

    - Header:  Manufacturer’s standard extruded aluminum header with a replaceable aluminum track extending full width of entrance unit.  Header to conceal door operators, carrier assemblies, and roller track; complete with hinged access panel for service of door operator, and controls.

- NOTE:  Consult SpecDesk for transoms and oversized packages

      - Header Span:   Maximum 16’-0” (4.9 m) without intermediate supports when entrance glazed with 1/4-inch glass.

        - Capacity:  Capable of supporting active breakout leafs up to maximum of 300 lb (136 kg) per leaf when header is supported per manufacturer’s recommendations.

      - Header Size:  4-1/2 inches (114.3 mm) wide by 7 inches (177.8 mm) high.

        - Header height including the sensor plate cap which spans the clear door opening width is 8 inches (203.2 mm) high.

      - Header Access:  Continuous hinge at top of header allows cover to swing and allow complete access to operator and internal electronic and mechanical assemblies.

      - Design:  Closed header when doors in closed position.

  - HARDWARE

    - Hardware:  Provide manufacturer’s standard hardware as required for operation indicated.

      - Breakaway arms and bottom pivot assemblies shall be supplied by the manufacturer and shall be adjustable to comply with applicable codes.

- NOTE:  Magnetic catches are optional and replace the standard ball catch.

      - [Magnetic catch(s) to retain breakout door and sidelite panels in the closed position.]

- NOTE:  Consult SpecDesk for locking hardware options

      - [Locking hardware shall be provided as indicated.]

        - Electrified slide lock shall automatically lock the sliding function of all sliding door panels within the entrance when the door panels are in the closed position.

          - [Fail secure operation:  Slide lock shall lock the sliding function of the door panels upon loss of power.]

          - [Fail safe operation:  Slide lock shall unlock the sliding function of the door panels upon loss of power.]

          - Exterior jamb mounted key switch to unlock sliding door operation.

    - Guide Track/Threshold:  Manufacturer’s threshold as indicated.

- NOTE:  Option 1 is the manufacturer’s standard threshold/track for the clean room entrance and Option 2 is optional – full breakout entrance

      - [Full Breakout Entrance Guide Track:  Recessed floor mounted aluminum guide track(s) adjacent to the sidelite portion of the sliding automatic door assembly.]

      - [Full Breakout Entrance Threshold:  1/2 inch (12.7 mm) high continuous aluminum threshold with integral track shall span the width of the sliding door header and fit between the vertical framing members.  Threshold design shall allow for optional extruded ramps to securely interlock to flat section to meet ADA requirements.]

        - [Surface mounted threshold with interlocking ADA accessible ramps.]  [Recessed mounted threshold.]

- NOTE:  Option 3 is the only threshold/track available for the clean room entrance – fixed sidelite entrance

      - [Fixed Sidelite Entrance Guide Track:  Aluminum guide track integrated in the bottom of the sidelite portion of the sliding automatic door assembly.]

- NOTE:  Option 4 is the manufacturer’s standard threshold/track for the clean room entrance and Option 5 is optional - surface mounted entrance

      - [Surface Mounted Entrance Guide Track:  Floor mounted guide track(s) mounted adjacent to the wall construction.  The track(s) shall not extend into the clear door opening.]

        - [Surface mounted track.]  [Recessed mounted track.]

      - [Surface Mounted Entrance Guide Track:  Aluminum fixed sidelite guide track mounted along the face of the wall.  The track shall not extend past the jamb into the door opening.]

  - DOOR OPERATORS AND CONTROLS

    - Door Operator and Controller:

      - Electro-mechanical controlled unit utilizing a high-efficiency, energy efficient, DC motor requiring a maximum of 3 amp current draw, allowing 5 operators on one 20 amp circuit. The supplied system shall have the capability to operate at full performance well beyond a brown out and high line voltage conditions (85V – 265V) sensing changes and adjusting automatically.  The operator shall allow an adjustable hold open time delay of 0 to 60 seconds and have internal software to incorporate a self-diagnostic system.

      - Operating Temperature Range: -31 F to 130 F (-35 C to 54.44 C).

    - Microprocessor Control Box:

      - Modular control unit to allow for changing technology.  Factory-adjusted configuration with opening and closing speeds set to comply with ANSI/BHMA A156.10 requirements and electronic dampening to reduce wear on drive train. Should the drive train operations deviate from design criteria ranges, Watchdog Control Circuit Monitoring will assume command of the system and shut down the automatic function allowing a secondary supervisory circuit to perform as a backup.  Control unit shall allow the following functions:

        - Diagnostics with the ability to produce application data.

      - Mode Selector Control:

- NOTE:  The standard mode selector control is the rotary knob switch

        - [Multi-position  [rotary knob]  [keyed cylinder]  mode selector control shall allow selection of the indicated functions to be engaged when switch is turned to the appropriate setting.]

        - [Touch pad mode selector control with the following visual indication and trouble shooting.]

          - Touch pad mode selector with selection indication, to allow selection of the indicated functions.

          - Touch pad security code to prevent accidental change of settings.

          - Multi-colored, trouble shooting LED indicator for the following conditions:  inspection is required, service is required, or error condition such as door in breakout position.

- NOTE:  The standard mounting location is jamb mounted

        - Mode Selector Control Mounting:  Control shall be mounted as indicated:

          - [Jamb mounted.]

          - [Remote mounted.]

          - [Header cover mounted.]

        - Mode selector control to allow the following functions:

          - “Off”

          - “Exit Only” one way traffic with automatic operation from the interior.

          - “Two Way Traffic” allowing automatic operation from exterior and interior.

          - “Partial Opening” energy saving door position allows door to automatically adjust opening width based on amount of usage, that is, full open during high use and partial open during low use. The control for this setting is programmable allowing adjustment to both the usage setting and the opening width.

          - “Hold Open” doors activated and held in the full open position.

  - ACTIVATION AND SAFETY CONTROL DEVICES

    - General:  Provide the types of activation and safety devices specified in accordance with ANSI/BHMA standards, for the condition of exposure and for long-term, maintenance-free operation under normal traffic load for type of occupancy indicated.  Coordinate activation and safety devices with door operation and door operator mechanisms.

    - Combination Activation Motion Sensor/Safety Presence Sensor:

      - Shall be a sliding door sensor utilizing K-band microwave technology to detect motion and focused active infrared technology to detect presence, combined in a single housing surface mounted on each side of the header.

        - Presence sensor shall remain active at all times.

        - The sensor shall communicate with the automatic door operator through a self-monitoring connection that allows the door to go into a fail-safe mode preventing the door from closing in the event of a sensor failure.

      - Motion/presence detecting sensors to be field installed and adjusted.

- NOTE:  Consult Besam for “Knowing Act Activation” device options.  Ex: push plates

  - ELECTRICAL

    - High-Efficiency DC Motor: Maximum of 3 amp current draw, allowing 5 operators to run on one 20 Amp circuit.

    - Power:  Self-detecting line voltage capable control. 120 VAC through 240 VAC, 50/60 Hz, 3 amp minimum incoming power with solid earth ground connection for each door system.

    - Key Impulse Input:  Input for card readers or remote activation with independent adjustable hold open delay.

    - Wiring:  Separate internal channel raceway free from moving parts.

    - Brown out / high voltage capability: System has capability to operate at full performance well beyond brown out and high voltage line conditions (85 V – 265 V) sensing changes and adjusting automatically.

    - [Convenience Battery: Shall be concealed in header and capable of full operation with blackout conditions, including sensor capabilities for minimum of 100 cycles.]

    - [Digital Cycle Counter:  Battery powered, 7 digit LCD cycle counter with a reset feature to track door usage cycles.]

  - ALUMINUM FINISHES

- NOTE:  Consult SpecDesk for custom finish options

    - Comply with NAAMM's "Metal Finishes Manual for Architectural and Metal Products" for recommendations for applying and designating finishes.

    - [Anodized Finish:]

      - [AAMA 611, Clear, AA- M12C22A41, Class I, 0.018 mm.]

      - [AAMA 611, Dark Bronze, AA-M12C22A44, Class I, 0.018 mm.]

      - [AAMA 611, Custom anodized to match architect’s sample.]

    - [Painted Finish:]

      - [Powder coat painted to match architect’s sample.]

      - [Kynar finish, [2 coat]  [3 coat], to match architect’s sample.]

    - [Clad Finish (protective coatings by others):]

      - [Stainless steel with #4 satin finish.]

      - [Stainless steel with #8 mirrorlike, reflective, non-directional finish.]

      - [Bronze with a satin finish.]

      - [Bronze with a polished, non-directional finish.]

      - [Brass with a satin finish.]

      - [Brass with a polished, non-directional finish.]

    - [To match architects sample.]

- EXECUTION

  - EXAMINATION

    - Examine doors and frames, with Installer present, for compliance with requirements for installation tolerances, wall and floor construction, and other conditions affecting performance.

    - Examine roughing-in for electrical source power to verify actual locations of wiring connections.

    - Proceed only after such discrepancies or conflicts have been resolved.

  - INSTALLATION

    - Do not install damaged components.  Fit frame joints to produce hairline joints free of burrs and distortion. Rigidly secure non-movement joints.

    - Entrances:  Install automatic entrances plumb and true in alignment with established lines and grades without warp or rack of framing members and doors.  Anchor securely in place.

      - Install surface mounted hardware using concealed fasteners to greatest extent possible.

      - Set headers, carrier assemblies, tracks, operating brackets and guides level and true to location with anchorage for permanent support.

    - Door Operators:  Connect door operators to electrical power distribution system as specified in Division 26 Sections.

    - Glazing:  Glaze sliding automatic entrance door panels in accordance with the Glass Association of North America (GANA) Glazing Manual, published recommendations of glass product manufacturer, and published instructions of automatic entrance system manufacturer.

    - Sealants:  Comply with requirements specified in division 7 Section “Joint Sealants” to provide a weather tight installation.

      - Set thresholds, bottom guide and track systems and framing members in full bed of sealant.

      - Seal perimeter of framing members with sealant.

    - Signage:  Apply signage on both sides of each door and sidelite as required by ANSI/BHMA A156.10 and manufacturers installation instructions.

  - ADJUSTING

    - Adjust door operators, controls and hardware for smooth and safe operation and for weather tight closure. Adjust doors in compliance with ANSI/BHMA A156.10.

    - Verify installation and alignment of all entrance weather-stripping as required for compliance with specified air infiltration requirements.

  - FIELD QUALITY CONTROL

    - Before placing doors into operation, AAADM certified technician shall inspect and approve doors for compliance with ANSI/BHMA A156.10. Certified technician shall be approved by the manufacturer.

  - CLEANING AND PROTECTION

    - Clean adjacent surfaces soiled by door installation.

    - Clean glass and metal surfaces promptly after installation. Remove excess sealants, compounds, dirt and other substances. Repair damages to match original finish.

  - DEMONSTRATION

    - Engage a factory-authorized representative to train Owner's maintenance personnel to adjust, operate, and maintain safe operation of the door.

END OF SECTION