SECTION 08 71 13 – AUTOMATIC DOOR OPERATORS

- GENERAL

  - SUMMARY

- NOTE:   Edit this specification as required for the project

- NOTE:  This specification is for automatic door operators complying with ANSI/BHMA A156.10 for power operated pedestrian door applications.

    - This section includes the following types of automatic door operators:

      - Full energy power door operators for swinging doors.

    - Related Sections:

      - Division 7 Sections for caulking to the extent not specified in this section.

      - [Division 8 Section “Aluminum-Framed Entrances and Storefronts” for entrances furnished separately in Division 8 Section.]

      - [Division 8 Section “Sliding Automatic Entrances” for single and bi-parting sliding automatic entrance doors with sidelites.]

      - [Division 8 Section “Door Hardware” for hardware to the extent not specified in this Section.]

      - Division 26 and 28 Sections for electrical connections including conduit and wiring for automatic entrance door operators and access control devices.

  - REFERENCES

    - References:  Refer to the version year adopted by the Authority Having Jurisdiction.

      - ANSI A117.1 - Accessible and Usable Buildings and Facilities.

      - ICC/IBC - International Building Code.

      - CUL – Approved for use in Canada.

      - NFPA 70 - National Electrical Code.

      - NFPA 80 - Fire Doors and Windows.

      - NFPA 101 - Life Safety Code.

      - NFPA 105 - Installation of Smoke Door Assemblies.

    - American National Standards Institute (ANSI) / Builders Hardware Manufacturers Association (BHMA).

      - ANSI/BHMA A156.10 American National Standard for Power Operated Pedestrian Doors.

    - Underwriters Laboratories (UL).

      - UL10C – Positive Pressure Fire Tests of Door Assemblies.

      - UL 325 Standard for Safety for Door, Drapery, Gate, Louver and Window Operators and Systems.

      - UL991 - Tests for Safety-Related Controls Employing Solid-State Device.

- NOTE:  Delete the following if not applicable

    - [Canadian Standards Association (CSA).]

      - CAN/CSA-C22.2 NO 223-M91 –  Power Supplies With Extra-Low-Voltage Class 2 Outputs.

      - CAN/CSA-C22.2 NO 223-M92 – Operators and Systems of Doors, Gates, Draperies, and Louvers.

    - American Association of Automatic Door Manufacturers (AAADM).

    - American Society for Testing and Materials (ASTM).

      - ASTM B221 Standard Specification for Aluminum and Aluminum Alloy Extruded Bars, Rods, Wire, Profiles and Tubes.

      - ASTM B209 Standard Specification for Aluminum and Aluminum Alloy Sheet and Plate.

    - American Architectural Manufacturers Association (AAMA).

      - AAMA 611 Voluntary Specification for Anodized Architectural Aluminum.

    - National Association of Architectural Metal Manufacturers (NAAMM).

      - Metal Finishes Manual for Architectural Metal Products.

    - International Code Council (ICC).

      - [IBC:  International Building Code.]

      - [CBC: California Building Code.]

  - DEFINITIONS

    - Activation Device: Device that, when actuated, sends an electrical signal to the door operator to activate the operation of the door.

      - Knowing act:	Consciously initiating the opening of a power operated door using acceptable methods including wall mounted switches such as push plates and controlled access devices such as keypads, card readers and key switches.

    - Safety Device: A device that detects the presence of an object or person within a zone where contact could occur and provides a signal to stop the movement of the door.

    - Double Egress Doors:  A pair of doors that swing with the two doors moving in opposite directions and no mullion between them.

  - PERFORMANCE REQUIREMENTS

    - Automatic door equipment accommodates medium to heavy pedestrian traffic.

    - Opening Force Requirements for Egress Doors: In the event of power failure to the operator, swinging automatic entrance doors shall open with a manual force, not to exceed 30lbf (133N) applied at 1” (25 mm) from the latch edge of the door.

    - Closing Time:

      - Doors shall be field adjustable to close from 90 degrees to 10 degrees in 2 seconds or longer as applicable per ANSI/BHMA A156.10 standards.

      - Doors shall be field adjusted to close from 10 degrees to fully closed in not less than 1.5 seconds.

  - SUBMITTALS

    - Product Data:  Manufacturer's product data sheets including installation details, material descriptions, dimensions of individual components and profiles, fabrication, operational descriptions and finishes.

    - Shop Drawings:  Submit manufacturer’s shop drawings, including elevations, sections and details, indicating dimensions, materials, operator, motion /presence sensor control device, anchors, hardware, finish, options and accessories.

      - Indicate required clearances, and location and size of each field connection.

      - Indicate locations and elevations of entrances showing activation and safety devices.

      - Wiring Diagrams:  For power, signal, and activation / safety device wiring.

    - Samples: Submit manufacturer’s samples of aluminum finish.

    - Informational Submittals:  Manufacturer's product information and applicable sustainability program credits that are available to contribute towards a LEED rated project certification.

      - Credit MR 4: Manufacturer's or fabricator's certificate indicating percentage of post-consumer recycled content by weight and pre-consumer recycled content by weight for each Product specified under this Section.

      - EPDs:  Manufacturer’s Environmental Product Declaration.

    - Manufacturers Field Reports:  Submit manufacturer’s field reports from AAADM certified technician of inspection and approval of doors for compliance with ANSI/BHMA after completion of installation.

    - Operating and Maintenance Manuals:  Provide manufacturers operating and maintenance manuals for each item comprising the work of this section in quantity as required in Division 01, Closeout Submittals.  The manual to include the name, address, and contact information of the manufacturers providing the operators and their nearest service representatives.  The final copies delivered after completion of the installation test to include spare parts list.

    - Warranties and Maintenance:  Special warranties and maintenance agreements specified in this Section.

  - QUALITY ASSURANCE

    - Manufacturers Qualifications:  Engage qualified manufacturers with a minimum 10 years of documented experience in manufacturing of doors and equipment of similar to that indicated for this Project and that have a proven record of successful in-service performance.  Manufacturer to have a company certificate issued by AAADM.

    - Installer Qualifications:  Installers, trained by the primary product manufacturers, with a minimum 3 years documented experience installing and maintenance of units similar in material, design, and extent to that indicated for this Project and whose work has resulted in construction with a record of successful in-service performance.

    - Certified Inspector Qualifications: Certified by AAADM.

    - Source Limitations for Automatic Door Operators:  Obtain each type of operator and sensor components specified in this Section from a single source, same manufacturer unless otherwise indicated.

    - Certifications: Operators shall be certified by the manufacturer to meet performance design criteria in accordance with the following standards.

      - ANSI/BHMA A156.10 American National Standard for Power Operated Pedestrian Doors.

      - NFPA 101 - Life Safety Code.

      - UL 325 - Standard for Door, Drapery, Gate, Louver, and Window Operators and Systems.

      - Labeled that operator has been tested for use on fire rated doors by a third-party certification agency.

    - Emergency Exit Door Requirements: Comply with requirements of authorities having jurisdiction for automatic entrance doors serving as a required means of egress.

  - COORDINATION

    - Coordinate door operators with doors, frames and related work to ensure proper size, thickness, hand, function and finish. Coordinate hardware for automatic entrances with hardware required for rest of the project.

      - Review sequence of operation for doors with automatic operators as noted in the hardware schedule in Division 8 Section “Door Hardware”.

    - Electrical System Roughing-in: Coordinate layout and installation of power door operators with connections to power supplies, fire alarm system, smoke evacuation system, and access control system as applicable.

  - WARRANTY

    - General Warranty:  Reference Division 01, General Requirements.  Special warranties specified in this Article shall not deprive Owner of other rights Owner may have under other provisions of the Contract Documents and shall be in addition to, and run concurrent with, other warranties made by Contractor under requirements of the Contract Documents.

    - Automatic Door Operators shall be free of defects in material and workmanship for a period of one (1) year from the date of substantial completion.

    - During the warranty period a factory-trained technician shall perform service and affect repairs. An inspection shall be performed after each adjustment or repair.

    - During the warranty period all warranty work, including but not limited to emergency service, shall be performed during normal business hours.

    - Manufacturer shall have in place a dispatch procedure that shall be available 24 hours a Day, 7 Days a week for emergency call back service.

- PRODUCTS

  - MANUFACTURER

    - Manufacturer:  ASSA ABLOY Entrance Systems, 1900 Airport Road, Monroe, NC 28110. Toll Free (877) SPEC-123.  Fax (704 ) 290- 5555 Website www.assaabloyentrance.us contact: specdesk.na.entrance@assaabloy.com

- NOTE:  Revise the following substitution clause as required by project requirements.  Select either Item “B” or “C”

    - [Substitutions:  Requests for substitution and product approval in compliance with the specifications must be submitted in writing and in accordance with the procedures outlined in Division 1, Section “Substitution Procedures”.  Approval of requests is at the discretion of the architect, owner, and their designated consultants.]

    - [Substitutions:  Not Permitted.]

  - MATERIALS

    - Aluminum:  Alloy and temper recommended by manufacturer for type of use and finish indicated, as indicated below:

      - Extruded Aluminum, Alloy 6063-T5.

  - SWING DOOR OPERATORS

    - Model:  ASSA ABLOY SW300 full energy automatic door operator (Basis of Design):

      - Reference Standard:  ANSI/BHMA A156.10.

      - Configuration:  Operator to control single swinging doors and pairs of swinging doors as indicated on the drawings and specified below:

        - Traffic Pattern:  [One way.]  [Two way.]

        - [Pairs of Doors:  Simultaneous swing.]

        - [Double Egress Doors:  Simultaneous swing.]

      - Automatic Door Operator:  Electro-mechanical, non-handed operator, powered by 24 volt, 1/4 hp motor.  Operator shall be adjustable to compensate for different manual push forces as required.

        - [Surface Mounted Operator:]

          - Side Access Operator Housing:  Operator is contained in 6-1/2” (166.1 mm) deep x 2-3/4“ (70 mm) high extruded aluminum housing with a removable cover.

          - Surface Mounted Housing:  Continuous for full width of door.

          - Connecting Hardware: Surface mounted operators to have a steel arm from the operator, mounted to the top face of the swing door.

          - Surface mounted operator shall be capable of operating and controlling up to a 621 pound (281.68 kg) door, 48 inches (1219 mm) in width.

          - Labeled that operator has been tested for use on fire rated doors by a third-party certification agency.

        - Operator shall be field switchable between an ANSI/BHMA A156.10 and an ANSI/BHMA A156.19 compliant operator and vice versa.  Addition of the activation devices may be required to comply with the applicable standard.

        - Operator Temperature Range:  Capable of operating within temperature ranges of -31 F to 160 F (-35 C to 71 C).

        - Electrical Characteristics:  Maximum power consumption is 300 watts (2.5 amps at 120 VAC), 50/60hz, built-in thermal overload protection.

        - [Battery Convenience Mode:  Operator to maintain continuous operation by battery power during power failure. Battery is continuously monitored and provides a warning signal if the battery is not working properly.]

        - [Digital Cycle Counter:  Battery powered, 7 digit LCD cycle counter with a reset feature to track door usage cycles.]

      - Door Operation:

        - Opening Cycle:  The adjustable speed operator mechanically powers the drive shaft and the torque control maintains constant speed throughout the opening cycle regardless of stack pressures or wind speed.  Operator shall allow manual door operation with operational forces as indicated to fully open the door applied at 1” (25 mm) from the latch edge of the door.

          - Manual push force shall be adjustable from 5 lbf to 30 lbf maximum.

        - Hold Open:  The operator shall stop and hold the door open at the selected door opening angle for an adjustable period of time (1.5 seconds to 30 seconds).

        - Closing Cycle:  Spring close with speed controlled power assist.

          - Upon loss of power, dynamic braking will control the door insuring controlled closing.

          - Selectable Torque Control:  Automatically adjusts torque without changing the closing speed of the operator.

            - When the torque control is activated, the closing speed shall remain constant regardless of stack pressures or wind speed.

            - Torque Cancellation:  The torque control is deactivated whenever there is a signal received from door mounted sensors.

            - The torque control is disabled during manual use of the door.

        - Wind Force Dampening:  The operator electromechanically counteracts wind forces, slowing down the door movement to safely open or close the door.

        - Stack Pressure Compensation:  Operator shall counteract positive stack pressures, negative stack pressures, and sudden changes of stack pressures.  The operator never allows the door to open or close faster than the speed control settings, regardless of pressures.

        - Obstruction Control:  The operator will stop and reverse the door movement.

        - Electric Lock Management:

          - Internal module for electrified locking integration.

          - Electric Lock Output:  Selectable 12 VDC, maximum 1200 mA / 24 VDC, maximum 600 mA.

          - Lock monitoring prevents operator(s) from opening door(s) until release of electrified lock.

          - Operator pulls door closed before opening, automatically unjamming electric latch hardware.

          - Sequenced operation between operators for pairs of doors allowing lock release and astragal coordination.

        - Lock Retry Circuit:  If attempt to fully close the door is unsuccessful, the operator will automatically reverse open 10 degrees and reclose in an attempt to successfully close the door.

        - Selectable Alarm Reset:  The operator can be field set so that after receiving an alarm signal, the operator will not accept any activation impulses and will operate only as a manual door closer until manually reset.

        - Electronic Controls:  Solid state integrated circuit controls the operation and switching of the swing power operator.  The electronic control provides low voltage power supply for all means of actuation.  The controls include time delay (1 to 30 seconds) for normal cycle.

        - Control Switch: Automatic door operators shall be equipped with the following type of multi-position function switch:

- NOTE:  Select one of the following control switch options

          - [3 position rocker switch mounted on end cap (On-Off-Hold).]

          - [2 position rocker switch mounted on end cap (On-Off).]

          - [3 position toggle switch remotely mounted (On-Off-Hold).]

          - [4 position rotary switch remotely mounted (On-Off-Hold-Special Function).]

      - Operator Interface:

        - Safety Sensor Integration for overhead presence safety device and door mounted reactivation safety sensors.

  - [ACTIVATION BY SMOKE EVACUATION SYSTEM]

- NOTE:  Delete this entire option if not required  – Review operation with Architect

    - General:  Provide activation by the smoke evacuation system and/or fire detection system.  Coordinate other required activation devices and safety devices with door operation and door operator mechanisms.

    - Activation:	Smoke evacuation system and/or fire detection system shall provide activation of the operator by means of a normally open maintained contact to control the opening and closing of the door systems in the event of an alarm condition.  Doors are to be held open until the smoke evacuation/fire detection system is reset.

  - ACTIVATION DEVICES

- NOTE:  Select the type of activation devices and safety devices required for the project

    - General:  Provide activation devices in accordance with ANSI/BHMA standards, for condition of exposure and for long-term, maintenance-free operation under normal traffic load for type of occupancy indicated.  Coordinate activation and safety devices with door operation and door operator mechanisms.

- NOTE:  Select automatic activation devices if required for the project

- NOTE:  ANSI/BHMA A156.10 allows both “Automatic Activation Devices” and “Knowing Act Activation Devices”

    - [Automatic Activation Device:]

      - [Motion Sensors:  Self-contained, K-band-frequency, microwave-scanner unit; achieving both a narrow or wide sensing pattern, adjustable to provide detection field sizes and functions required by ANSI/BHMA A156.10.]

        - Motion Sensors to offer three motion detection actions; bi-directional, uni-directional, and uni-MTF detection.

- NOTE:  Secondary activation is required by ANSI/BHMA A156.10

    - [Secondary Activation Device:]

      - Secondary activation:  Where activation is by a “knowing act” device, provide a secondary activation sensor as required by ANSI/BHMA A156.10.

- NOTE:  Select the type of knowing act activation devices if required for the project

    - [Knowing Act Activation Device:]

      - [Push Plate Switch:  [4-1/2 inch square]  [4-1/2 inch round]  [6-inch round]  stainless steel faceplate engraved with blue "Push to Open" text and an ADA logo, hard wired.]

      - [Push Plate Switch:  Jamb mounted 1-1/2 inch x 4-1/2 inch, stainless steel faceplate engraved with blue "Push to Open" text and an ADA logo, hard wired.]

      - [Wireless Push Plate Switch:  Radio controlled,  [4-1/2 inch square] ]  [4-1/2 inch round]  [6 inch round]  stainless steel faceplate engraved with blue "Push to Open" text and an ADA logo.]

      - [Vestibule Push Plate Switch:  4-1/2 inch square with dual stainless steel faceplates and push plate switches, hard wired.  Each faceplate engraved with blue "Push to Open" text, an ADA logo, and directional arrows pointing in opposite directions.]

      - [Touchless Sensor Switch:  [Jamb mounted 1-1/2 inch x 4-1/2 inch]  [2-3/4 inch x 4-1/2 inch]  [4-1/2 inch square]  [black polycarbonate faceplate with white “Wave to Open” text and hand icon graphics,]  [white polycarbonate faceplate with black “Wave to Open” text and hand icon graphics,]  hard wired.  Microwave technology has an adjustable range of 4 inches to 24 inches.]

      - [Touchless Sensor Switch:  [Jamb mounted 1-1/2 inch x 4-1/2 inch]  [2-3/4 inch x 4-1/2 inch]  [4-1/2 inch square]  stainless steel faceplate, black “Wave to Open” text, black sensor plate with white hand icon graphics, hard wired.  Microwave technology has an adjustable range of 4 inches to 24 inches.]

      - [Vertical Activation Column:  Vertical activation column with activation of automatic door from any approach and height level along vertical profile.]

        - Size:  36” high x 6” wide with a maximum of 1-1/2” projection from wall.

        - Graphics:  Engraved with blue "Push to Open" text and an ADA logo.

        - Vandal resistant mounting.

        - Stainless Steel:  Type 304, nominal thickness 18/19 gauge sheet metal.

          - Stainless Steel Finish:  Satin US32D.

        - Hard Wire:  Hard-wired from flush junction box.

        - [Vertical Activation Column:  [“INGRESS’R” as manufactured by Wikk Industries, Inc.]  [“LPR36” as manufactured by BEA.]

- NOTE:  Retain the above “vertical activation column” or the following where push plates are required by code to be mounted at a high and low position

- NOTE:  Delete the following if vertical actvation column is retained above

      - [Mounting:  Push plates are to be mounted at both a high and low position at heights and locations as required by the building code.]

- NOTE:  Retain the following when card reader or keypad activation is required

    - [Card Reader Activation:] [Keypad Activation:]

      - [Card reader] [Keypad] to provide signal for unlatching of door hardware and subsequent activation of door operator.

      - [Card reader] [Keypad] to activate the operation of the push plate.

        - The push plate provides signal to control unlatching of door hardware and subsequent activation of door operator.

        - [No push plate operation on secure side of entrance; activation signal by access control device.]

      - Access control systems, including card readers and/or keypads, shall be provided by others.

  - SAFETY DEVICES

    - General:  Provide safety devices in accordance with ANSI/BHMA A156.10 standards, for condition of exposure and for long-term, maintenance-free operation under normal traffic load for type of occupancy indicated.  Coordinate safety devices with door operation and door operator mechanisms.

- NOTE:  Select from one of the following the presence detection systems – Consult SpecDesk for safety device recommendations

- NOTE:  Presence detection is required by ANSI/BHMA A156.10 to provide safety at the doorway

    - Presence Detection Systems and Safety Devices:

- OPTION 1:  Select the following safety sensor system for maximum safety at the doorway, compliant with ANSI/BHMA A156.10 requirements, and have a minimal footprint

      - [ASSA ABLOY I-Adapt Premium Safety Sensor System A202 (Basis of Design), Door Mounted Presence Sensor Adaptable Field (DMPS-AF) as specified:]

        - Door Mounted Presence Sensor Adaptable Field (DMPS-AF): Door mounted combination activation motion sensor/safety presence sensor.  Sensor shall be mounted on both the swing (pull) side and the approach (push) side of the door (2 sensors per leaf).

- OPTION 1:  Select one of the following sensor technologies – “active infrared presence technology” (Optex) is manufacturer’s default sensor;  “presence laser technology” (BEA) is a secondary option.  Sensors for both selections are mounted on the upper hinged corner of the door and have a minimal footprint.

          - [Sensor shall utilize active infrared presence technology with auto adapting field to detect moving or stationary presence of people or objects in the swing path of the door.]   [Sensor shall utilize presence laser technology with auto adapting field to detect moving or stationary presence of people or objects in the swing path of the door.]

          - Presence detection shall always be active and remain active when the door is in motion.  The sensor provides a full detection pattern that covers the entire swing of the door and also provides detection in the full open and full close position.

          - The sensor has an auto adapting field which maximizes the sensor pattern tailored to the environment providing detection beyond the moving part of the door.  The sensor provides detection to the wall and or guide rails ensuring maximum safety resulting in the door stopping much sooner (further) from the person using the doorway.

          - Upon detection the sensor shall provide a signal to slow, stop, or reverse the door action depending on the situation.

          - The sensor provides secondary activation as required for “knowing act” doorways.

          - Since the infrared presence detection is always active, no overhead safety sensor is required.

          - Motion/presence detecting sensors to be field installed and adjusted.

- NOTE:  Coordinate with the Architect if an EPT (electrical power transfer) is required in lieu of the standard door cord power transfer – EPT specified in Division 8 Section “Door Hardware”

        - Power transfer from the door mounted safety sensor to operator shall be [through an exposed door cord.]   [through an EPT (electrical power transfer) specified in Division 8 Section “Door Hardware”.]

- OPTION 2:  Select the following safety sensor system for safety at the doorway, compliant with ANSI/BHMA A156.10 requirements

      - [ASSA ABLOY I-Adapt Flex Safety Sensor System A102 (Basis of Design), Combination of an Overhead Presence Sensor (OPS) and Door Mounted Presence Sensors (DMPS) as specified:]

        - Overhead Presence Sensor (OPS):  Header mounted, overhead presence sensor utilizing infrared technology for detection; adjustable to provide detection field sizes and functions required by ANSI/BHMA A156.10.  Unit to provide the following two independently adjustable patterns of detection:

          - The door closed position covering the area on the swing side of the door.

          - The door open position including an area of detection that reaches through the threshold toward the non-swing side of the door.

          - The unit is not active during the door closing cycle.

        - Door Mounted Presence Sensor (DMPS):  Door mounted infrared presence safety device (mounted at top of each door); adjustable to provide detection field sizes and functions required by ANSI/BHMA A156.10.

          - The door mounted presence detector shall be mounted on both the swing (pull) side and the approach (push) side of the door (2 sensors per leaf), providing detection on both sides of the door.

          - Unit to provide detection during the travel of the door.

          - Upon detection the sensor shall provide a signal to stop or reverse the door action.

          - [Door mounted presence sensor shall be monitored, providing a supervisory circuit to the operator.]

- NOTE:  Coordinate with the Architect if an EPT (electrical power transfer) is required in lieu of the standard door cord power transfer – EPT specified in Division 8 Section “Door Hardware”

        - Power transfer from the door mounted safety sensor to operator shall be [through an exposed door cord.]   [through an EPT (electrical power transfer) specified in Division 8 Section “Door Hardware”.]

- OPTION 3:  Select the following safety sensor system for safety at the doorway, compliant with ANSI/BHMA A156.10 requirements

      - [ASSA ABLOY I-Adapt Flex Safety Sensor System A101 (Basis of Design), Combination of an Overhead Presence Sensor (OPS) and a Door Mounted Presence Sensor (DMPS) as specified:]

        - Overhead Presence Sensor (OPS):  Header mounted, overhead presence sensor utilizing infrared technology for detection; adjustable to provide detection field sizes and functions required by ANSI/BHMA A156.10.  Unit to provide the following two independently adjustable patterns of detection:

          - The door closed position covering the area on the swing side of the door.

          - The door open position including an area of detection that reaches through the threshold toward the non-swing side of the door.

          - The unit is not active during the door closing cycle.

        - Door Mounted Presence Sensor (DMPS):  Door mounted infrared presence safety device (mounted at top of each door); adjustable to provide detection field sizes and functions required by ANSI/BHMA A156.10.

          - The door mounted presence detector shall be mounted on the swing (pull) side of the door (1 sensor per leaf), providing detection on one side of the door only.

            - On “knowing act” double egress doorways, the door mounted presence detector shall be mounted on the approach (push) side of the door (1 sensor per leaf).

          - Unit to provide detection during the travel of the door.

          - Upon detection the sensor shall provide a signal to stop or reverse the door action.

          - [Door mounted presence sensor shall be monitored, providing a supervisory circuit to the operator.]

- NOTE:  Coordinate with the Architect if an EPT (electrical power transfer) is required in lieu of the standard door cord power transfer – EPT specified in Division 8 Section “Door Hardware”

        - Power transfer from the door mounted safety sensor to operator shall be [through an exposed door cord.]   [through an EPT (electrical power transfer) specified in Division 8 Section “Door Hardware”.]

  - [ACCESSORIES]

- NOTE:  ANSI/BHMA A156.10 requires guide rails for power operated swing doors unless the door is located adjacent to a wall – Consult SpecDesk for guide rail requirements - delete this section if not required

    - [Guide Rails:  [Anodized aluminum]  [Painted aluminum]  [Stainless steel], fabricated from  [bars]  [tubing], minimum 30 inches (762 mm) high, and finished to match doors unless otherwise indicated; positioned and projecting from face of door jamb for distance as indicated, but not less than that required by ANSI/BHMA A156.10 for type of door and direction of travel; with filler panel.]

      - Filler Panel:  [Expanded aluminum mesh.]  [Polycarbonate plastic.]

        - Orient expanded aluminum mesh with long dimension of diamonds  [parallel to top rail.]  [perpendicular to top rail.]  [horizontal.]  [vertical.]

        - Color: [To match guide rails.]  [Clear.]  [As indicated by manufacturer's designations.]  [Match Architect's sample.]

      - Mounting:  [Jamb and floor.]  [Floor, freestanding.]  [In-ground, freestanding.]

- NOTE:  If the following is retained, provide details

      - [Guide Rails:  To be as detailed (see architectural drawings).]

      - Guide Rail Finishes:

        - [Anodized Finish:]

          - [AAMA 611, Clear, AA- M12C22A41, Class I, 0.018 mm.]

          - [AAMA 611, Dark Bronze, AA-M12C22A44, Class I, 0.018 mm.]

          - [AAMA 611, Custom anodized to match architect’s sample.]

        - [Painted Finish:]

          - [Powder coat painted to match architect’s sample.]

          - [Kynar finish, [2 coat]  [3 coat], to match architect’s sample.]

        - [Stainless Steel Finish:]

          - Stainless steel with a satin finish.

        - [To match architects sample.]

- NOTE:  Delete this section “Bollard Post” if not required for mounting activation devices

    - Bollard Post:  Bollard post shall be [anodized extruded aluminum, nominal 1/8” (4 mm) wall thickness.] [painted extruded aluminum, nominal 1/8” (4 mm) wall thickness aluminum.] [stainless steel tube, type 304, satin finish, .120” (3 mm) wall thickness.]

      - Post Height: [42” high surface mounted post complete with anchoring base and anchors.] [54” height in-ground post complete with anchors.]

        - Post Mounting: [Surface mounted with exposed base.] [Surface mounted with concealed mounting base.] [In-ground mounting.]

      - Post Size:  Nominal [4-1/2” x 4-1/2” square post.]  [4” diameter post.]  [6” x 6” square post.]  [6” diameter post.]

        - [Square posts to have 1/64” radiused corners.]

        - [Round posts to have adapter cuff for mounting of flat type push plate and card reader.]

      - Post Top:  [Flat top, removable black plastic cap that allows RF transmission.]  [Flat top, removable stainless steel cap.]  [Angled top, removable black plastic cap that allows RF transmission.]  [Angled top, welded.]  [Dome top, welded.]

      - Push Plate:  See Section “ACTIVATION DEVICES” for push plate and card reader requirements.

      - Bollard Post Finish:

        - [Anodized Finish:]

          - [AAMA 611, Clear, AA- M12C22A41, Class I, 0.018 mm.]

          - [AAMA 611, Dark Bronze, AA-M12C22A44, Class I, 0.018 mm.]

          - [AAMA 611, Custom anodized to match architect’s sample.]

        - [Painted Finish:]

          - [Powder coat painted to match architect’s sample.]

          - [Kynar finish, [2 coat]  [3 coat], to match architect’s sample.]

        - [Stainless Steel Finish:]

          - Stainless steel with a satin finish.

        - [To match architects sample.]

        - [Finished to match entrance.]

  - ALUMINUM FINISHES

- NOTE:  Consult SpecDesk for custom finish options

    - Comply with NAAMM's "Metal Finishes Manual for Architectural and Metal Products" for recommendations for applying and designating finishes.

    - Automatic Door Operator Enclosure:

      - [Anodized Finish:]

        - [AAMA 611, Clear, AA- M12C22A41, Class I, 0.018 mm.]

        - [AAMA 611, Dark Bronze, AA-M12C22A44, Class I, 0.018 mm.]

        - [AAMA 611, Custom anodized to match architect’s sample.]

      - [Painted Finish:]

        - [Powder coat painted to match architect’s sample.]

        - [Kynar finish, [2 coat]  [3 coat], to match architect’s sample.]

      - [Clad Finish (protective coatings by others):]

        - [Stainless steel with #4 satin finish.]

        - [Stainless steel with #8 mirrorlike, reflective, non-directional finish.]

        - [Bronze with a satin finish.]

        - [Bronze with a polished, non-directional finish.]

        - [Brass with a satin finish.]

        - [Brass with a polished, non-directional finish.]

      - [To match architects sample.]

- EXECUTION

  - EXAMINATION

    - Examine doors and frames, with Installer present, for compliance with requirements for installation tolerances, wall and floor construction, and other conditions affecting performance of swinging power operated doors.

    - Examine roughing-in for electrical source power to verify actual locations of wiring connections.

    - Proceed only after such discrepancies or conflicts have been resolved.

  - INSTALLATION

    - Do not install damaged components.  Fit joints to produce hairline joints free of burrs and distortion. Rigidly secure non-movement joints.

    - Operators:  Install automatic door operators plumb and true in alignment with established lines and grades without warp or rack of framing members and doors.  Anchor securely in place.

      - Install surface mounted hardware using concealed fasteners to greatest extent possible.

      - Set headers, carrier assemblies, tracks, operating brackets and guides level and true to location with anchorage for permanent support.

    - Door Operators:  Connect door operators to electrical power distribution system [including smoke evacuation system and/or fire detection system] as specified in Division 26 Sections.

    - Sealants:  Comply with requirements specified in division 7 Section “Joint Sealants” to seal between the operator housing and the adjacent surfaces.

    - Signage:  Apply signage on both sides of each door and sidelite as required by ANSI/BHMA A156.10 and manufacturers installation instructions.

  - ADJUSTING

    - Adjust automatic door operators, controls and hardware for smooth and safe operation and for weather tight closure. Adjust doors in compliance with ANSI/BHMA A156.10.

  - FIELD QUALITY CONTROL

    - Before placing doors into operation, AAADM certified technician shall inspect and approve doors for compliance with ANSI/BHMA A156.10.  Certified technician shall be approved by manufacturer.

  - CLEANING AND PROTECTION

    - Clean adjacent surfaces soiled by automatic door operator installation.

    - Clean metal surfaces promptly after installation. Remove excess sealants, compounds, dirt and other substances. Repair damages and finish to match original finish.

  - DEMONSTRATION

    - Engage a factory-authorized representative to train Owner's maintenance personnel to adjust, operate, and maintain safe operation of the door.

END OF SECTION